PERSPECTIVES DESCRIPTION

As a general note the inputs and outputs are not filtered, each perspective will show the inputs and outputs on its entirety.

- VAVDD-CD Damper Control:

Displays modules associated with the cooling control and a CD Flow calculation widget useful to verify calculation of the flow rate.

- VAVDD-HD Damper Control:

Displays modules associated with the heating control and a HD Flow calculation widget useful to verify calculation of the flow rate.

- VAVSD-Damper Control:

Displays the modules associated with damper control under any configuration. Contains a widget that encapsulates the supply flow calculation objects.

- AHU-Cooling Control:

Shows the modules associated with cooling control for any AHU staged or chilled water configuration.

- AHU-Heating Control:

Displays the modules associated with heating control for any AHU staged, hot water /steam or gas coil configuration.

- AHU-Econ Damper Control:

Displays the modules associated with the economizer damper control for any economizer configuration.

- AHU-Fan Control:

Shows associated fan control modules for supply, return and relief fans when selected.

- Fan Coil-Cooling Control:

Displays the modules associated with the fan coil cooling control for any configuration.

- Fan Coil-Heating Control:

Displays the modules associated with the fan heating control for any configuration.

- HP-Cooling Control:

Displays the modules associated with the heat pump cooling control.

- HP-Heating Control:

Displays the modules associated with the heating control and supplemental heat when selected.

- Module-Logic:

Displays the modules in the upper window and the logic/system information in the lower portion of the window. This enables the user to observe the modules and the logic at the same time. Ensure the Freeze Selection box is un-checked; this will give you the ability to click on any logic modules and see its logic below.

- UV-Cooling Control:

Displays the modules associated with the unit ventilator cooling control for ASHRAE Cycles 1,2,3 and W.

- UV-Heating Control:

Displays the modules associated with the unit ventilator heat control for ASHRAE  Cycles 1,2,3 and W.

- VAVSD-Heating Control:

Displays the modules associated with the box heating control and supplemental heating control when selected. Contains two widgets that displays the box heating and supplemental heating tuning information outputs.